# Beech model 18 CHECKLIST

# Pre-Start Checklist

1.  Cabin Door	CLOSED

2.  Controls	FREE

3.  Landing Gear SW.	DOWN

4.  Parking Brake	ON

5.  Bat. & Gen. SWS.	ON

6.  Gear Safe Light	CHECK

7.  Fuel Quantity	CHECK

8.  Fuel Selectors	MAIN

9.  Crossfeed	OFF

10.  Anti-Ice Fluid	CHECKED

11.  Trim	SET > T/O

12.  Flap Switch	NEUTRAL

13.  Throttles	1/8 OPEN

14.  Props.	HIGH

15.  Manifold Heat	COLD

16.  Oil Shutters	HOT

17.  Oil Shutoff Valves	OPEN

18.  Circuit Breakers	CHECKED

19.  Electrical Switchs	OFF

20.  Alt. Static Source	NORMAL

21.  Oil By-Pass Valves	AS REQUIRED

22.  Warning Lights	CHECKED

# Start Checklist

1.  Fuel Boost Pumps	ON

2.  Magnetos	ON

3.  Engine Sel. SW.	RT or LT

4.  Ignition Boost SW.	ON

5.  Starter (2 Revs)	ENGAGE

6.  Mixtures	RICH

7.  Primer	AS REQUIRED

(SAME PROC. FOR OTHER ENGINE)

8.  Ignition Boost SW.	OFF

9.  Fuel Boost Pumps	OFF

10.  Warm Up Engines	1000-1200 RPM

#### After-Starting Engines

1.  Oil Pressure	UP

2.  Fuel Boost Pumps	OFF

3.  Lights	AS REQUIRED

4.  Radios	ON

5.  Tailwheel	UNLOCKED

6.  Parking Brake	UNLOCKED

# Run-Up

1.  Temps. & Press.	GREEN ARC

2.  Props (1500 rpm)	EXERCISE

3.  Manifold Heat	CHECK

4.  Vacuum	CHECK

5.  Volt/Ammeters	CHECK

6.  Mags. (1800 rpm)	CHECK

## Before Take-Off

1.  Controls	FREE

2.  Mixture	RICH

3.  Props.	HIGH RPM

4.  Friction Locks	SET

5.  Oil Shutters	AS REQUIRED

6.  Manifold Heat	COLD

7.  Flaps 	UP

8.  Fuel Selector	MAIN TANKS

9.  Oil By-Pass	CLOSED

10.  Trim	SET

11.  D.G.’s	SET

12.  Instruments	CHECK

13.  Navs.	CHECK / SET

Beech Model 18 CHECKLIST

# Take-Off

1.  Time	RECORD

2.  Fuel Boost Pumps	ON

3.  Landing Lights	ON

4.  Inverter	ON

5.  Transponder	ALT

6.  Tail Wheel	LOCKED

7.  D.G.’s	R.W. HEADING

8.  Pitot Heat	AS REQUIRED

9.  Throttles 	36.5” & 2300 RPM

10.  Oil Pressure	GREEN ARC

# 11.  Oil Temperature	GREEN ARC

# Climb

1.  Gear	UP

2.  Flaps	UP

3.  Manifold Pressure	32”

4.  Prop. RPM	2150 RPM

5.  Fuel Boost Pumps	OFF – PX CK

# 6.  Landing Lights	OFF

# 7.  Airspeed	120/140 MPH

# 8.  Flight Plan	OPEN

# Cruise

1.  Manifold Pressure	28”

2.  Prop. RPM	2000 RPM

3.  Mixture	Lean

4.  Oil Shutters	150 F.

5.  Manifold Heat	AS REQUIRED

6.  Fuel Quantity	Manage

# Approach / Descent Checklist

1.  Customs Code	OBTAIN

2.  Fuel Selectors	MAIN TANKS

3.  Mixtures	ENRICHEN

4.  Manifold Pressure	26”

5.  Manifold Heat	AS REQUIRED

6.  Altimeters	SET

7.  Navs	SET

8.  Flight Plan	CLOSED

# Before Landing Checklist

1.  Fuel Boost Pumps	ON

2.  Landing Lights	ON

3.  Props.	2150 RPM

4.  Landing Gear	DOWN/GREEN

5.  Manifold Heat	OFF

6.  Heater	CYCLE

7.  Brakes	CHECK

8.  Flaps	AS NEEDED

# After Landing Checklist

1.  Fuel Boost Pumps	OFF

2.  Landing Lights	OFF

3.  Inverter	OFF

# 4.  Transponder	STBY

5.  Props.	HIGH RPM

6.  Oil Shutters	150 F.

7.  Flaps	UP

8.  Pitot Heat	OFF

9.  Taxi Lights	AS REQUIRED

10.  Time	RECORD

# Shutdown Checklist

# 1.  Parking Brake	SET

2.  Radios	OFF

3.  Mixtures	IDLE CUT-OFF

4.  Magnetoes	OFF

5.  Battery/Gen SW’s	OFF

6.  Tailwheel	LOCK

7.  Controls	LOCK

8.  Tiedowns	(3 POINTS)

9.  Chocks	ALL WHEELS

Airspeeds – Miles-Per-Hour

Vne 257	Vyse 122	Vmc 86

Vno 205	Vyxe 110	Vle 160

Vy 135	Vlo 160	Vx 119

Va 153	Vfe 120	Vso 75

Vs 80	Max T\O Wt:  10,100

Max Lnd Wt:    9,800