NORMAL CHECK LIST FOR EMB-110

This Check List is real; however do not use it on real world!
NOTE: Absent items must be jumped.

BEFORE START (THROUGH FLIGHT)

COCKPIT PREPARATION			COMPLETED
EXTERNAL INSPECTION			COMPLETED
CHOCKS / TAIL SUPORT			CKD
BATTERY VOLTAGE			CHECKED
FASTEN BELTS / NO  SMOKING		ON
FIRE TEST				CHECKED
CVR TEST				CHECKED
INSTRUMENTS				SET/CKED
RADIOS / RADAR / XPDR			SET/STBY
FUEL TOTALIZER				CHECKED
FUEL QUANTITY				LBS
POWER LEVERS				MIN.
PROPELLER  LEVERS			MAX/RPM

CLEARED TO START

CHOCKS / TAIL SUPORT			ON BOARD
DOORS LIGHTS				CLOSED/CKED
PARKING BRAKE				APPLIED
BATTERY SELECTOR			INT / EXT
ROTATION BEACON			ON
NAVIGATION LIGHTS			AS RQD
INTER / CABIN LIGHT			AS RQD
INVERTERS 2 AND 1			ON
AUX. HYDRAULIC PUMP			ON
FORWARD WINDOWS			CLOSED
FUEL PUMPS				L / R  ON

AFTER START

BATTERY SELECTOR			INTERNAL
GENERATOR				ON
VOLT / AMPER				CHECKED
PX / PITOT HEAT				AS RQD
BATTERY  TEMPERATURE			CHECKED
RADAR					STBY
AR CONDITIONING			AS RQD
STEERING				ON

TAXING

TAXI LIGHT				ON
NORMAL BRAKES				CHECKED
MULT. ALARM PANEL (MAP)		CHECKED
INSTRUMENTS				CHECKED
*FUEL SISTEM				CHECKED
FLIGHT CONTROLS			CLEAR
TRIM CONTROLS				3 SET
*ENGINE TEST				CKD
AUTOFEATHER SWITCH			ARM
TAKE OFF DATA				CHECKED

BEFORE TAKE-OFF

INERTIAL SEPARATOR			AS RQD
LANDING LIGHTS				ON
TRANSPONDER				ALT
FLAPS					25%
AR CONTIDIONING			VENT / OFF 
FUEL LEVERS				HIGH
CHRONOMITS				ON

AFTER TAKE-OFF

LANDING GEAR				UP/NO LIGHTS
FLAPS					UP
ALTIMETERS				1013 HPA
AUTO FEATHER				OFF
AUX. HYDR. PUMP			OFF
CLIMB POWER				ADJUSTED
PROPELLER				91% NH
INERTIAL SEPARATOR			OFF
AR CONDITIONING			AS RQD
ENGINE INSTRUMENTS			CHD
LANDING / TAXI LGHTS			OFF
FASTEN BELTS				AS RQD
ALTIMETERS				1013 HPA

CRUISE

FASTEN BELTS				AS RQD
PX / PITOT HEAT				ON/OFF
PROPELLER LEVERS			83`%NH
POWER LEVERS				ADJUSTED

BEFORE DESCENT

LANDING DATA				COMP/SET
APPROACH BRIEFING			COMPLETED
FASTEN BELTS				AS RQD
POWER 					ADJUSTED

APPROACH – 10 MILES OUT

FASTEN BELTS / NO SMOKING		ON
LANDING LIGHTS			AS RQD
AUTO FEATHER					ARMED
PROP-SYNC				OFF
AUX. HYDR. PUMP			ON/CHECKED
ALTIMETERS				(QNH)SET

BEFORE LANDING

RADAR					STBY
LANDING GEAR				DOWN/3GREEN
BRAKES / HYDR. PRESSURE		CHECKED
PROPELLER LEVERS			91%NH
FLAPS					CHECKED
AIR CONDITIONING			VENT/OFF
STEERING				ON
INERTIAL SEPARATOR			AS RQD
LANDING AND TAXI LIGHTS		AS RQD

AFTER LANDING

LANDING LIGHTS				OFF
INERTIAL SEPARATOR			AS RQD
AUTO FEATHER				OFF
RADIOS / TRANSPONDER			SET/STBY
FUEL LEVERS				LOW
FLAPS					25%
TRIM CONTROLS				SET

SHUTDOWN

PARKING BRAKE				APPLIED
POWER LEVERS				MIN
AR CONDITIONING			OFF
PROPELLER LEVERS			FEATHER
FUEL LEVERS				CUT-OFF
RADIO / RADAR / MAP LGTS		OFF
STEERING				OFF
AUX. HYDR. PUMP			OFF
FUEL PUMPS (MAN / AUX)		OFF
PROPELLER LEVERS			MAX/RPM
PANEL / INTER / EXTER LIGHTS		OFF
GENERATORS				OFF
BATTERY				OFF
PARKING BRAKE				RELEASED

(**) These items only apply when the aircraft is abandoned

** FWD WINDOWS			CLOSED
** PROP. BLOCKADE / COVERS		APPLIED
** DOORS				CLOSED