Ilyushin IL-18D

ESPAÑOL / ENGLISH

PROCEDIMIENTOS____________________________________________________

Antes de partir

- Combustible – SEGÚN

- Puerta principal – ABRIR

- Freno de parqueo - PONER

- Todos los interruptores - OFF

- Planta en tierra – ON (Si no existe, arrancar APU)

- Batería - ON

- Luces Panel, Cabina pilotaje, Cabina pasajeros - SEGUN

- Nav, No fumar, Cinturones - ON

Arranque

- Beacon - ON

- Puerta principal - CERRAR

- Limpia parabrisas, Separador inercial - SEGUN

- Mandos Gases - MINIMA

- Energía del panel de motores - ON

- Cerrar línea de combustible del motor 4 - ON

- Partida Motor 4 - ENCENDIDO

- Chequear Fuel Flow motor 4 - ESTABLE

- Con Motor 4 arrancado, Generador 4 - ON

- (Repetir proceso con resto de motores)

- Todos los interruptores aviónica y sistemas – ON

Rodaje a pista activa

- Luces Taxi – SEGÚN

- Frenos de parqueo – LIBERAR

- Flaps – 15º

Carrera y Despegue

- Limpia parabrisas – OFF

- Separador inercial - SEGUN

- Landing - ON

- Acelerar RPM = 100%

- Rotar ~ 230 Km/h IAS

- Despegar ~ 260 Km/h IAS

Ascenso

- Climb positivo recoger trenes y chequear - ARRIBA

- A 280 Km/h IAS y más 300 mts Flaps – 0º

- Luces Taxi, Landing - OFF

- Ajustar potencia ~ 350 Km/h IAS

- Máximo régimen de ascenso -10 mts/seg

Crucero

- Máximo permitido – 10000 mts

- Velocidad crucero ~ Mach 0.60 ~ 400 Km/h IAS

Descenso

- Reducir potencia

- Máximo descenso - 10 mts/seg

- Ajustar potencia

Aproximación

- Ajustar potencia ~ 350 Km/h IAS

- A 1000 mts nivelar hasta - 320 Km/h IAS

- Por debajo de 320 Km/h IAS Flap - 15º

- Por debajo de 300 Km/h IAS Flap - 30º

- Continuar aproximación ~ 280 Km/h IAS

- Luces Taxi, Landing - ON

- Trenes - ABAJO

Aterrizaje

- Velocidad final ~ 260 Km/h IAS

- Con aterrizaje asegurado gases – MINIMA

- Con trenes en pista reversos – PONER

- A menos de 200 Km/h IAS frenos – SEGUN

- A menos de 50 Km/h IAS reversos - QUITAR

- Frenos - SEGUN

- Flaps - 0%

- Landing - OFF

Rodaje a rampa

- Limpia parabrisas, Separador inercial - SEGUN

- Beacon – ON

Apagado

- Freno de parqueo - PONER

- Gases - MINIMA

- Combustible - CERRAR

- Con motores apagados puerta principal - ABRIR

- Todos los interruptores – OFF

PROCEDURES_________________________________________________________

Before starting

- Load fuel – AT WILL

- Main door – OPEN

- Parquin brakes - LOOCK

- All switches - OFF

- Power Plant – ON (If not exist, start APU)

- Battery - ON

- In door lights – AT WILL

- Nav, No smoke, Seat belts - ON

Starting

- Beacon - ON

- Main door - CLOSE

- Windshield cleaner, Inercial Separator – AT WILL

- Throttle - MINIMAL

- Engines panel power  - ON

- Fuel flow key to engine 4 - ON

- Engine 4 starter - START

- Check engine 4 Fuel Flow gauge - ESTABLE

- With engine 4 running, Power generator 4 - ON

- (Repeat the starting prosses with remain engines)

- All avionics an system switches – ON

Rolling to active runway

- Taxi Lights – AT WILL

- Parking breaks – RELEASE

- Flaps – 15º

Takeoff running

- Windshield cleaner – OFF

- Inercial separator – AT WILL

- Landing lights - ON

- Full throttles RPM = 100%

- Rotate ~ 230 Km/h IAS

- Takeoff ~ 260 Km/h IAS

Ascenso

- Positive climb retrat gears and check - UP

- At 280 Km/h IAS and over 300 mts Flaps – 0º

- Taxi, Landing Lights - OFF

- Adjust speed ~ 350 Km/h IAS

- Max. climb rate -10 mts/seg

Cruise

- Max. Altittude – 10000 mts

- Cruise speed ~ Mach 0.60 ~ 400 Km/h IAS

Descend

- Reduce speed

- Max. descend rate - 10 mts/seg

- Adjust speed

Approximation

- Adjust speed ~ 350 Km/h IAS

- At 1000 mts level to reduce speed - 320 Km/h IAS

- Below 320 Km/h IAS Flap - 15º

- Below 300 Km/h IAS Flap - 30º

- Continue the app. ~ 280 Km/h IAS

- Taxi, Landing Lights - ON

- Gears - DOWN

Landing

- Final speed ~ 260 Km/h IAS

- With secure landing throttle  – MINIMAL

- With three points touch revers – ENGAGE

- Below 200 Km/h IAS brakes– ALL WILL

- Below 50 Km/h IAS revers - DESENGAGE

- Breaks – AT WILL

- Flaps - 0%

- Landing Lights - OFF

Taxing to ramp

- Windshield cleaner, Inertial Separator – AT WILL

- Beacon – ON

Shutdown

- Parking brakes - LOOCK

- Throttles - MINIMAL

- All fuel flow key - OFF

- With engines off main door - OPEN

- All switches – OFF