The SM.93 two-seat dive/torpedo bomber, also powered by the 1475 h.p. Daimler-Benz DB 605 twelve-cylinder vee engine, was unusual in that the pilot was accommodated in a prone position to prevent blacking-out in pulling out of dives. Bomb load was 3200 lb. (1800 lb. below the fuselage and 1400 lb. under the wings); armament comprised one 20-mm. cannon firing through the airscrew hub, one 12.7-mm. machine gun in each wing, and a third 12.7mm. gun fired by the observer. Loaded weight was 12,125 lb. Performance included a maximum speed of 360 m.p.h. at 22,960 ft., a cruising speed of 314 m.p.h., and a maximum diving speed of 590 m.p.h. Range was 994 miles and service ceiling 32,800 ft. Climb to 13,120 ft. took 5 min. 10 sec. Dimensions were: span 45 it. 7 1/4 in., length 36 ft. 1 3/4 in., height 12 ft. 5 1/2 in., and wing area 334.7 sq. ft. The sole prototype was tested by the Luftwaffe in 1943 before being abandoned at that critical stage of the war.